CSC111 Assembly Language Programming

Fall 2025 Midterm Exam

Student Name:                              ID：                              _s

| Total Points |  |
| --- | --- |

Note: This exam paper should be kept confidential and any dissemination violates copyright.

| Q1 | Q2 | Q3 | Q4 | Q5 | Q6 | Q7 | Q8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| /10 | /10 | /20 | /10 | /10 | /10 | /20 | /10 |

(In all the problems, the system is assumed to be 32-bit unless specified otherwise.)

Q1 Multiple-choice questions: enter your answer keys here:

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For the following multiple-choice questions, each question has exactly one correct answer key. If multiple choices are correct, choose the option “All of the above”. Fill in the answer keys in the table above. (Answer keys written in the question area will not be counted.)

- Which registers are general‑purpose on ARM Cortex‑M?
A. R0–R12
B. R13–R15
C. CONTROL, BASEPRI, PRIMASK
D. xPSR only

- What are the roles of R13, R14, and R15 on ARM Cortex‑M?
A. General purpose
B. Program counter, stack pointer, and link register in that order
C. Stack pointer (R13), link register (R14), program counter (R15)
D. Interrupt mask registerse

- On ARM Cortex-M3, the borrow and carry flags relation is:
A. Carry = Borrow
B. Carry = NOT Borrow
C. Borrow always 0
D. Carry always 0

- In two’s complement, TC(x) can be obtained by:
A. Invert bits
B. Invert bits and subtract one
C. Invert bits and add one
D. Add one then invert bits

- In a 5-bit system, which statement is true about −16 (10000₂)?
A. Its two’s complement is 00000₂
B. Its two’s complement is itself
C. It cannot be represented
D. It equals +16

- What is the bit width of each register in ARM Cortex-M processors?
A) 16 bits
B) 24 bits
C) 32 bits
D) 64 bits

- In ARM assembly instruction format, what is typically the first operand (operand1)?
A) Source register
B) Immediate value
C) Destination register
D) Memory address

- When adding two 64-bit integers split across two registers each, which instruction pair correctly handles the low and high halves?
A. ADC for high halves, then ADDS for low halves to set carry
B. ADC for low halves, then ADDS for high halves
C. ADDS for low halves to set carry, then ADC for high halves
D. ADD for low halves, then ADD for high halves

- In ARM subtraction, what does the carry flag C indicate when a borrow occurs in SUBS?
A. C = 1 when there is a borrow
B. C = 0 when there is a borrow
C. C toggles regardless of borrow
D. C is always preserved from the previous instruction

- Which single instruction multiplies a register by 17 using the barrel shifter on the second operand?
A. ADD r4, r4, r4, LSL #4
B. RSB r5, r5, r5, LSL #5
C. ADD r1, r0, r0, ASR #3
D. MUL r1, r0, #17

Q2. (10 points) Assuming a 4-bit system. Show the equivalent decimal values when the data is interpreted as unsigned binary or signed binary.

| Binary Value | Signed Decimal Value | Unsigned Decimal Value |
| --- | --- | --- |
| 1111 |  |  |
| 1011 |  |  |
| 0110 |  |  |
| 1001 |  |  |
| 0011 |  |  |

Q3. (20 points)

(a) (5 points) Assume an 8-bit system. Copy bits 5..3 in R4 to be the rightmost bits of R5, and set the other R5 bits to 0.

(b) (5 points) Assume an 8-bit system. Copy the bottom four bits 3..0 in R0 to bits 6..3 in R1, and keep the other bits in R1 unchanged.

(c) (10 points) Assume R0 and R1 are initialized with the values below. Write a sequence of assembly instructions that would produce the values in registers R2–R5, from R0 and R1.

Given:

| R0 | 0xCAFEFADE |
| --- | --- |
| R1 | 0x00FF00FF |

Produce:

| R2 | 0xCAFFFAFF |
| --- | --- |
| R3 | 0xCAFFFA8F |
| R4 | 0xFFFFFFCA |
| R5 | 0xDEFF00FF |

Q4. (10 points) Assume a 4-bit system. For each of the following operations, compute the result and NZCV flags, based on the first row that has been given to you. s

| Operation | Result in binary | Equivalent unsigned arithmetic in decimal | Equivalent signed arithmetic in decimal | NZCV |
| --- | --- | --- | --- | --- |
| 1001 + 0010 | 1011 | 9 + 2 = 11 | -7 + 2 = -5 | 1000 |
| 1101 + 1100 |  |  |  |  |
| 1101 - 1100 |  |  |  |  |
| 1100 + 1010 |  |  |  |  |
| 0100 - 0110 |  |  |  |  |
| 0100 + 0010 |  |  |  |  |

Q5. (10 points) For each of the following instructions (executed individually, not sequentially), compute the result and NZCV flags. Assume initially R0=0xFFFFFFFF, R1=0x00000001.

| Instruction | Result in dest. register R0 | NZCV |
| --- | --- | --- |
| MOVS R0, #0 |  |  |
| ANDS R0, #0 |  |  |
| ORRS R0, R0, R1 |  |  |
| ANDS R0, R0, R1 |  |  |
| ADDS R0, R0, R1 |  |  |
| SUBS R0, R0, R1 |  |  |
| ADDS R0, R0, R1, LSL #31 |  |  |
| BICS R0, R0, R1, LSL #31 |  |  |
| EORS R0, R0, R1, LSL #31 |  |  |
| LSLS R0, R0, #31 |  |  |

Q6. (10 points)

(a) Suppose r0 = 0x00008000, and the following memory layout:

| Address | Data |
| --- | --- |
| 0x00008007 | 0x79 |
| 0x00008006 | 0xCD |
| 0x00008005 | 0xA3 |
| 0x00008004 | 0xFD |
| 0x00008003 | 0x0D |
| 0x00008002 | 0xEB |
| 0x00008001 | 0x2C |
| 0x00008000 | 0x1A |

(a1) ARM processors can be configured as big-endian or little-endian. What is the value of r1 after running LDR r1, [r0]?

  - If little-endian, r1 = _________________________

  - If big-endian, r1= _________________________

(a2) Suppose the system is little-endian. What are the values of r1 and r0? Each instruction is executed individually, not sequentially.

- LDR r1,[r0,#4]

- LDR r1,[r0],#4

- LDR r1,[r0,#4]!

After LDR r1,[r0,#4]

r0 = _________________________

r1 = _________________________

After LDR r1,[r0],#4

r0 = _________________________

r1 = _________________________

After LDR r1,[r0,#4]!

r0 = _________________________

r1 = _________________________

(b) Assume little-endian memory ordering. Suppose r0 = 0x2000,0000 and r1 = 0x12345678.  All bytes in memory are initialized to 0x00. Fill the following table after the assembly program has finished execution.

STR r1, [r0], #4 STR r1, [r0, #4]! STR r1, [r0]

| Memory Address | Initial Memory Content | Final Memory Content |
| --- | --- | --- |
| 0x2000,0013 | 00 |  |
| 0x2000,0012 | 00 |  |
| 0x2000,0011 | 00 |  |
| 0x2000,0010 | 00 |  |
| 0x2000,000F | 00 |  |
| 0x2000,000E | 00 |  |
| 0x2000,000D | 00 |  |
| 0x2000,000C | 00 |  |
| 0x2000,000B | 00 |  |
| 0x2000,000A | 00 |  |
| 0x2000,0009 | 00 |  |
| 0x2000,0008 | 00 |  |
| 0x2000,0007 | 00 |  |
| 0x2000,0006 | 00 |  |
| 0x2000,0005 | 00 |  |
| 0x2000,0004 | 00 |  |
| 0x2000,0003 | 00 |  |
| 0x2000,0002 | 00 |  |
| 0x2000,0001 | 00 |  |
| 0x2000,0000 | 00 |  |

Q7. (20 points) Show all updates to registers as the assembly code shown below runs, assuming big-endian ordering. (Memory addresses increase from left to right in the table.) Fill in the tables of final register values and memory contents. Show the NZCV flags after execution, assuming NZCV=0000 initially.

Assembly program

LDR R1, =0x10000010 LDR R2, [R1] LDR R3, [R1, #4]! ASR R4, R2, #12 MOVW R5, #14 LSL R6, R5, #9 BIC R7, R2, R6 AND R8, R7, R6 STR R7, [R1, #8]

Initial memory contents:

| 0x10000010 | FF | EF | CD | AB | 00 | 00 | CD | AB | AA | BB | CC | DD | EE | FF | 11 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Final register values:

| R1 |  | R2 |  | R3 |  | R4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R5 |  | R6 |  | R7 |  | R8 |  |

Final memory contents:

| 0x10000010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Final NZCV = _________

Q8. (10 points) Consider an array[] of 25 integers (each integer is 4 bytes). Assume register R2 holds the base memory address of array[]. A compiler associates variables x and y with registers R0 and R1, respectively. Translate this C program into ARM assembly language based on the provided comments..

| C program | Assembly program |
| --- | --- |
| uint32_t x = array[5] + y; array[6] = x * 4; array[7] = x / 4; array[8] = x - 10; array[9] = x * (x - 1); | ____________________________; load array[5] (with 5*4 = 20 bytes offset) into ; temporary register R3 ____________________________; x = array[5] + y ____________________________; R3 = x * 4 ____________________________; store into array[6] ____________________________; R3 = x / 4 ____________________________; store into array[7] ____________________________; R3 = x – 10 ____________________________; store into array[8] ____________________________; R4 = x – 1 ____________________________; R3 = x * (x - 1) ____________________________; store into array[9] |

Appendix. Conversion table. (You may tear off and discard this page.)

| Decimal | Binary | Hex |
| --- | --- | --- |
| 0 | 0000 | 0x0 |
| 1 | 0001 | 0x1 |
| 2 | 0010 | 0x2 |
| 3 | 0011 | 0x3 |
| 4 | 0100 | 0x4 |
| 5 | 0101 | 0x5 |
| 6 | 0110 | 0x6 |
| 7 | 0111 | 0x7 |
| 8 | 1000 | 0x8 |
| 9 | 1001 | 0x9 |
| 10 | 1010 | 0xA |
| 11 | 1011 | 0xB |
| 12 | 1100 | 0xC |
| 13 | 1101 | 0xD |
| 14 | 1110 | 0xE |
| 15 | 1111 | 0xF |