No, you cannot simply replace MOVS r0, r0, LSL #1 with MOV r0, r0, LSL #1 if you want to use the carry flag for bit counting, because:

MOVS shifts and updates the condition flags, including the carry flag (which captures the leftmost bit shifted out).

MOV just copies/moves the value without updating any status flags (Carry, Zero, Negative, etc.).

Without status flag updates, the carry flag won't track the bit shifted out, so the later ADC instruction would not add the correct carry bit.

Thus, for counting bits by carry from shifts, MOVS is required to update the carry flag each time shifting left.

The plain MOV can only move data without affecting flags and is insufficient for this purpose.

No, you cannot replace ADC r2, r2, #0 with ADD r2, r2, #0 if you want to add the carry bit from the previous operation.

Why?

ADC (Add with Carry) adds two operands plus the carry flag.

ADD only adds the two operands without considering the carry flag.

Since the bit count relies on the carry flag (set by the previous MOVS shift left), replacing ADC with ADD would ignore the carry bit, causing incorrect counting.

In ADC r2, r2, #0, the immediate #0 means the register r2 plus 0 plus carry are summed, effectively adding carry to r2.

Using ADD r2, r2, #0 adds zero and ignores carry, so no bit would be counted from the carry.

Hence, keep using ADC to accumulate the bits from the carry flag properly.