# Ch16 Timer PWM Quiz ANS

1. What is the primary disadvantage of using Polling instead of Interrupts?

A. Polling is more complex to implement.

B. Polling wastes CPU cycles by continuously checking for events.

C. Polling cannot handle multiple devices.

D. Polling requires more hardware wiring than interrupts.

ANS: B

2. In the ARM Cortex-M architecture, what is the role of the Nested Vectored Interrupt Controller (NVIC)?

A. To generate clock signals for the CPU.

B. To coordinate multiple interrupt sources, manage priorities, and enable/disable interrupts.

C. To automatically save all General Purpose Registers (R0-R15) to flash memory.

D. To convert analog signals to digital signals for processing.

ANS: B

3. Where is the Interrupt Vector Table typically located in memory (before any relocation)?

A. 0x20000000

B. 0x08000000

C. 0x00000000

D. 0xFFFFFFFF

ANS: C (0x00000000 is the typical start address)

4. Which specific set of registers is automatically pushed onto the stack (Stacking) when an interrupt occurs?

A. R0–R12, SP, LR, PC

B. R0–R3, R12, LR, PC, xPSR

C. R4–R11, SP

D. All CPU registers (R0–R15)

ANS: B

5. During the Stacking process, the Stack Pointer (SP) is:

A. Incremented by 32 bytes.

B. Decremented by 32 bytes (for a full descending stack).

C. Incremented by 4 bytes.

D. Left unchanged; registers are saved to a special buffer.

ANS: B (Decremented by 32 bytes for 8 registers x 4 bytes)

6. What value does the Link Register (LR) hold when the processor is executing an Interrupt Service Routine (ISR)?

A. The address of the next instruction in the main program.

B. A special EXC_RETURN value (e.g., 0xFFFFFFF9) indicating how to return.

C. 0x00000000

D. The address of the ISR itself.

ANS: B (EXC_RETURN value)

7. Which Stack Pointer is selected by default at Reset?

A. Process Stack Pointer (PSP)

B. Main Stack Pointer (MSP)

C. Floating Point Stack Pointer

D. System Stack Pointer

ANS: B (MSP is selected at reset)

8. If SPSEL bit in the Control Register is 0, which stack pointer is used in Thread Mode?

A. MSP

B. PSP

C. Both MSP and PSP simultaneously

D. None of the above

ANS: A (MSP is used if SPSEL = 0)

9. To exit an ISR and return to the main program, which instruction is typically used?

A. MOV PC, #0

B. BX LR

C. POP {R4-R11}

D. RET

ANS: B (BX LR triggers the unstacking process)

10. Which register is used to divide the CPU clock frequency to generate a slower Timer Clock frequency?

A. Auto-Reload Register (ARR)

B. Capture Compare Register (CCR)

C. Prescaler Register (PSC)

D. Control Register (CR1)

ANS: C (Prescaler Register)

11. If the CPU Clock is 16 MHz and PSC is set to 15999, what is the Timer Clock frequency (fCK_CNT)?

A. 16 MHz

B. 1 MHz

C. 1 kHz

D. 100 Hz

ANS: C (16,000,000 / (15999 + 1) = 1,000 Hz = 1 kHz)

12. In "Up-counting Mode," what happens when the Counter reaches the value in the Auto-Reload Register (ARR)?

A. It stops counting immediately.

B. It resets to 0 and generates an Update Event (UEV).

C. It reverses direction and starts counting down.

D. It waits for a software reset trigger.

ANS: B (Resets to 0 and generates UEV)

13. The formula for the Timer Period in Up-counting mode is:

A. (ARR + 1) * Clock Period

B. ARR * Clock Period

C. (PSC + 1) * ARR

D. 2 * ARR * Clock Period

ANS: A

14. In "Center-aligned Counting Mode," the counter counts:

A. Up from 0 to ARR, then resets to 0.

B. Down from ARR to 0, then resets to ARR.

C. Up from 0 to ARR, then Down from ARR to 0.

D. Randomly between 0 and ARR.

ANS: C

15. Which PWM Mode behaves as "Low True" (Active Low) in Up-counting mode?

A. PWM Mode 1

B. PWM Mode 2

C. Center-aligned Mode

D. Capture Mode

ANS: A (Mode 1 is Low True/Active when CNT < CCR)

16. In PWM Mode 1 (Up-counting), if CNT < CCR, the output is:

A. Active (High)

B. Inactive (Low)

C. Toggled

D. Undefined

ANS: A

17. To increase the Duty Cycle in PWM Mode 1, you should:

A. Decrease the CCR value.

B. Increase the CCR value.

C. Set ARR to 0.

D. Increase the Prescaler (PSC).

ANS: B (Higher CCR means the "Active" condition CNT < CCR lasts longer)

18. How is the Duty Cycle calculated for PWM Mode 1?

A. Duty Cycle = 1 - (CCR / (ARR + 1))

B. Duty Cycle = CCR / (ARR + 1)

C. Duty Cycle = (ARR + 1) / CCR

D. Duty Cycle = PSC / ARR

ANS: B

19. A timer runs from a 72 MHz clock. PSC = 71 and ARR = 999. In PWM Mode 1, CCR = 250. What is the duty cycle?

A. 10%

B. 25%

C. 50%

D. 75%

ANS: B (Duty = CCR / (ARR + 1) = 250 / 1000 = 25%)

20. In center-aligned mode, a timer has ARR = 4000, PSC chosen so that the counter clock is 50 kHz. What is the PWM period?

A. 40 ms

B. 160 ms

C. 320 ms

D. 640 ms

ANS: B (Period = 2*ARR*T_clk=2*4000/50,000 = 0.16 s = 160 ms)