# Ch 2 Data Representation Quiz

- Which hex digit corresponds to the 4-bit pattern 1101?
A. 0xB
B. 0xC
C. 0xD
D. 0xE

ANS:

- Convert binary 10110 to decimal:
A. 20
B. 22
C. 18
D. 26

ANS:

- In a 5-bit system, adding unsigned ints 28 and 6 sets which condition?
A. No flags set
B. Carry flag set
C. Overflow flag set
D. Zero flag set

ANS:

- In a 5-bit system, subtracting unsigned ints 3 - 5 results in which carry/borrow status?
A. Carry=1 (Borrow=0)
B. Carry=0 (Borrow=1)
C. Carry=1 (Borrow=1)
D. Carry=0 (Borrow=0)

ANS:

- On ARM Cortex-M3, the borrow and carry flags relation is:
A. Carry = Borrow
B. Carry = NOT Borrow
C. Borrow always 0
D. Carry always 0

ANS:

- In two’s complement, TC(x) can be obtained by:
A. Invert bits
B. Invert bits and subtract one
C. Invert bits and add one
D. Add one then invert bits

ANS:

- In a 5-bit system, which statement is true about -16 (10000₂)?
A. Its two’s complement is 00000₂
B. Its two’s complement is itself
C. It cannot be represented
D. It equals +16

ANS:

- Signed overflow can occur when:
A. Adding operands with different signs
B. Subtracting operands with the same sign
C. Adding two negatives
D. Subtracting a negative from a negative

ANS:

- In CPSR after ADD/SUB, which flag denotes carry?
A. N
B. Z
C. C
D. V

ANS:

- In a 5-bit system, to compute a+b for a=0b10000 and b=0b10000, software should check which flag if a,b are unsigned vs. signed, respectively?
A. Unsigned→V, Signed→C
B. Unsigned→C, Signed→V
C. Unsigned→N, Signed→Z
D. Unsigned→Z, Signed→N

ANS:

- In a 5-bit system, the same binary addition can represent both unsigned 23+6=29 and signed −9+6=−3 because:
A. Adder interprets sign automatically
B. Two’s complement allows the same hardware; interpretation differs in software
C. Hardware selects mode via a pin
D. Only subtraction shares hardware

ANS:

- For char str = "ARM Assembly", what must the final byte be and what is the string’s size in Bytes?
A. 0x00; 13 
B. 0x20; 12 
C. 0x41; 13
D. 0x79; 12 (ASCII hex code for lowercase ‘y’ is 0x79)

ANS: