# Ch3 ARM ISA Quiz

1. Which ARM processor family is specifically designed for microcontroller applications and is cost-sensitive?
A) ARM Cortex-A family
B) ARM Cortex-R family
C) ARM Cortex-M family
D) ARM Cortex-X family

ANS:

2. What is the bit width of each register in ARM Cortex-M processors?
A) 16 bits
B) 24 bits
C) 32 bits
D) 64 bits

ANS:

3. Which registers are considered "Low Registers" in ARM Cortex-M and can be accessed by any instruction?
A) R0-R7
B) R8-R12
C) R13-R15
D) R0-R12

ANS:

4. What does the R15 register represent in ARM architecture?
A) Stack Pointer
B) Link Register
C) Program Counter
D) Status Register

ANS:

5. Which instruction set was first introduced in ARM7TDMI processors in 1995?
A) ARM32
B) Thumb-1 (16-bit ThumB)
C) Thumb-2
D) ARM64

ANS:

6. In ARM assembly instruction format, what is typically the first operand (operand1)?
A) Source register
B) Immediate value
C) Destination register
D) Memory address

ANS:

7. Which assembly directive is used to allocate one or more 32-bit words of data?
A) DCB
B) DCW
C) DCD
D) DCQ

ANS:

8. How many ENTRY directives must be present in an ARM assembly application?
A) Zero
B) Exactly one
C) At least one per source file
D) One per subroutine

ANS:

9. What does the EXPORT directive do in ARM assembly?
A) Imports symbols from other files
B) Declares a symbol and makes it visible to the linker
C) Defines the end of a procedure
D) Allocates memory space

ANS:

10. Which two stack pointers does ARM Cortex-M4 support?
A) MSP and LSP
B) PSP and LSP
C) MSP and PSP
D) SSP and USP

ANS:

11. Which directive pair is used to mark the start and end of a function in ARM assembly?
A) AREA and END
B) PROC and ENDP
C) ENTRY and EXIT
D) START and STOP

ANS:

12. What type of access do peripheral registers use in ARM Cortex-M processors?
A) Direct register access
B) Memory-mapped I/O
C) Port-mapped I/O
D) Interrupt-driven access

ANS:

13. Which assembly directive is used to give a symbolic name to a register?
A) EQU
B) RN
C) EXPORT
D) IMPORT

ANS:

14. What does the ALIGN directive accomplish in ARM assembly?
A) Sets register values to zero
B) Aligns data or code to a particular memory boundary
C) Imports external symbols
D) Defines constant values

ANS: