# Ch6 ARM Control Flow Quiz ANS

### Q1

Which condition flag is set to 1 when the result of an operation is zero?

- N (Negative flag)

- Z (Zero flag)

- C (Carry flag)

- V (Overflow flag)

Q1 ANS: B - The Z (Zero) flag is set to 1 when all bits of the result are 0.

### Q2

For unsigned subtraction, when is the Carry flag (C) set to 0?

- When no borrow occurs

- When a borrow occurs

- When the result is negative

- When overflow occurs

Q2 ANS; B - For unsigned subtraction, C = 0 (borrow = 1) when borrow takes place, meaning the true result < 0.

### Q3

The Overflow flag (V) is set to 1 when:

- Adding two different-signed numbers

- Adding two same-signed numbers produces opposite sign result

- Performing any multiplication

- Using MOV instruction

Q3 ANS: B - V = 1 when adding 2 same-signed numbers produces a result with the opposite sign (Positive + Positive = Negative, or Negative + Negative = Positive).

### Q4

What does the CMP R1, R2 instruction do?

- Saves the result of R1 - R2

- Performs R1 - R2 and discards the result, sets flags

- Performs R1 + R2 and sets flags

- Moves R2 to R1

Q4 ANS: B - CMP performs subtraction R1 - R2, same as SUBS, except the result is discarded (not written to destination register).

### Q5

The TST instruction performs which operation?

- R1 + R2

- R1 - R2

- R1 & R2 (bitwise AND)

- R1 ^ R2 (bitwise XOR)

Q5 ANS: C - TST performs bitwise AND on R1 and R2, same as ANDS, except the result is discarded.

### Q6

When using TEQ instruction, the Z flag is set to 1 when:

- R1 is greater than R2

- R1 and R2 are equal

- R1 is less than R2

- An overflow occurs

Q6 ANS: B - TEQ performs XOR operation. If R1 and R2 are equal, then R1 ⊕ R2 is zero, and Z is set to 1.

### Q7

Which instruction branches to a label and saves the return address in the link register (r14)?

- B label

- BX Rm

- BL label

- BLX Rm

Q7 ANS: C - BL (Branch with Link) copies the address of the next instruction into r14 (link register) and branches to the label.

### Q8

For signed comparison, which branch instruction is used for ‘greater than’?

- BHI

- BGT

- BHS

- BLO

Q8 ANS: B - BGT (Branch if signed Greater Than) is used for signed comparison, while BHI is for unsigned comparison.

### Q9

The BGE (Branch if signed Greater or Equal) instruction tests which condition?

- N = 1

- Z = 1

- N = V

- C = 1

Q9 ANS: C - BGE tests the condition N = V, which indicates signed greater than or equal.

### Q10

Which values represent 0xFFFFFFFF and 0x00000001 when interpreted as unsigned numbers?

- -1 and 1

- 2^32 - 1 and 1

- 1 and -1

- 255 and 1

Q10 ANS: B - When interpreted as unsigned 32-bit numbers, 2^32 – 1 = 4294967295 and 0x00000001 = 1.

### Q11

In the instruction ‘ADDLT r3, r2, r1’, when is the ADD operation executed?

- When N = V

- When N != V

- When Z = 1

- When C = 1

Q11 ANS: B - ADDLT (Add if signed Less Than) executes when N != V, indicating a signed less than condition.

### Q12

In a for loop implementation using ‘SUBS r1, r1, #1’ followed by ‘BNE loop’, what does this accomplish?

- Increments counter and loops if zero

- Decrements counter and loops if not zero

- Sets flags without changing register

- Unconditional branch

Q12 ANS: B - SUBS decrements the counter and sets flags, BNE branches back to loop if the result is not zero (Z=0).

### Q13

The CBZ instruction is equivalent to which sequence?

- CMP R1, #0; BNE label

- CMP R1, #0; BEQ label

- ADD R1, #0; BEQ label

- SUB R1, #0; BNE label

Q13 ANS: B - CBZ R1, label is equivalent to CMP R1, #0 followed by BEQ label, except CBZ doesn’t change status flags.

### Q14

For signed comparison after ‘CMP r0, r1’, if N=0 and V=0, what can be concluded?

- r0 < r1

- r0 >= r1

- r0 = r1

- Overflow occurred

Q14 ANS: B - When N=0 and V=0, there’s no overflow and the result is non-negative, so r0 - r1 >= 0, meaning r0 >= r1

### Q15

In ARM Thumb-2, what does ‘ITE’ stand for in IT blocks?

- If-Then-Else with 2 instructions

- If-Then-End

- Iterate-Then-Exit

- If-Test-Execute

Q15 ANS: A - ITE stands for If-Then-Else, allowing 2 following instructions where the first executes if condition is true, second if false.

### Q16

Which condition code tests for ‘unsigned higher’?

- HS

- HI

- GT

- GE

Q16 ANS: B - HI (unsigned HIgher) tests C=1 & Z=0, while HS tests C=1 (unsigned Higher or Same).

### Q17

In the assembly sequence ‘TEQ r0, #’!‘; TEQNE r0, #’?’’, what programming concept is being implemented?

- Logical AND

- Logical OR with short-circuit evaluation

- Nested if statements

- Switch statement

Q17 ANS: B - This implements logical OR (||) with short-circuit evaluation. The second TEQ only executes if the first test failed (NE condition).

### Q18

The BLS (Branch if unsigned Lower or Same) instruction tests which condition?

- C=0 or Z=1

- C=1 and Z=0

- N=V

- N!=V

Q18 ANS: A - BLS tests C=0 or Z=1, meaning either the carry is clear (indicating lower) or zero flag is set (indicating same).

### Q19

Which method is used to update condition flags with arithmetic operations?

- Always automatic

- Append ‘S’ to instruction or use compare instructions

- Use special flag register

- Only with branch instructions

Q19 ANS: B - Method 1: append ‘S’ (like ADDS) to update flags with result. Method 2: use compare instructions (CMP, CMN, TEQ, TST) that set flags only.

### Q20

In break and continue statements, what is the key difference in assembly implementation?

- Break uses BEQ, continue uses BNE

- Break branches out of loop, continue branches to loop increment/test

- Break sets flags, continue doesn’t

- No difference in assembly

Q20 ANS: B - Break branches completely out of the loop (to end), while continue branches to the loop’s increment/condition test part to start the next iteration.

## Quick Reference Summary

### Key Condition Codes:

- EQ/NE: Z=1 / Z=0 (Equal/Not Equal)

- LT/GE: N≠V / N=V (Signed Less Than/Greater Equal)

- GT/LE: Z=0 & N=V / Z=1 or N≠V (Signed Greater/Less Equal)

- LO/HS: C=0 / C=1 (Unsigned Lower/Higher Same)

- HI/LS: C=1 & Z=0 / C=0 or Z=1 (Unsigned Higher/Lower Same)

### Flag Setting Instructions:

- CMP: R1 - R2 (result discarded)

- TST: R1 & R2 (result discarded)

- TEQ: R1 ⊕ R2 (result discarded)

- CMN: R1 + R2 (result discarded)