ARM Cortex-M Subroutines: Complete Assessment

Comprehensive Multiple-Choice Quiz & Answer Key

Course: Embedded Systems with ARM Cortex-M Microcontrollers in Assembly Language and C
Topic: Chapter 8 - ARM Subroutines (Parameters, Stack Operations, Recursive Calls)
Total Questions: 20
Time Limit: 60-90 minutes

PART 1: MULTIPLE-CHOICE QUIZ

Question 1: Link Register (LR) Primary Function

What is the primary purpose of the Link Register (R14)?

A) To hold temporary data during arithmetic operations
B) To store the return address of a subroutine call
C) To hold the stack pointer value
D) To store interrupt status flags

Question 2: BL Instruction Behavior

When the BL (Branch and Link) instruction is executed, what two operations occur?

A) PC is set to target address; LR is incremented by 1
B) LR holds the target address; PC is incremented by 2
C) LR = PC + 4 (return address); PC = target address
D) SP is decremented; PC is set to target address

Question 3: Return from Subroutine

Which instruction is used to return control to the caller from a subroutine?

A) BL LR
B) BX LR
C) MOV PC, LR
D) JMP LR

Question 4: ARM Procedure Call Standard - Argument Passing (≤4 args)

According to the ARM EABI, how are the first four 32-bit arguments passed to a subroutine?

A) In registers R0-R3
B) On the stack
C) In registers R4-R7
D) In registers R0-R3 for first two, on stack for the rest

Question 5: 64-bit Argument Passing

How is a 64-bit argument (such as a long long) passed to a subroutine?

A) In a single 64-bit register
B) Split across R0 and R2
C) In two consecutive 32-bit registers (e.g., R0:R1 or R2:R3)
D) Always passed on the stack

Question 6: Extra Arguments (>4)

When a subroutine requires more than four arguments, where are the extra arguments passed?

A) In memory-mapped registers
B) On the stack by the caller
C) In the program counter
D) In the link register

Question 7: Return Value (32-bit)

Where is a 32-bit return value placed by a subroutine?

A) In register R0
B) In register R7
C) On the stack
D) In the link register

Question 8: Cortex-M Stack Convention

What stack convention does ARM Cortex-M use?

A) Full ascending stack (stack grows toward high memory)
B) Empty descending stack (stack grows toward low memory)
C) Full descending stack (SP points to last item pushed; stack grows toward low memory)
D) Empty ascending stack

Question 9: PUSH Instruction Equivalence

PUSH {register_list} is equivalent to which instruction?

A) STMIA SP!, {register_list}
B) STMDB SP!, {register_list}
C) LDMIA SP!, {register_list}
D) LDMDB SP!, {register_list}

Question 10: POP Instruction Equivalence

POP {register_list} is equivalent to which instruction?

A) LDMIA SP!, {register_list}
B) STMIA SP!, {register_list}
C) LDMDB SP!, {register_list}
D) STMDB SP!, {register_list}

Question 11: Register Order in PUSH

When you execute PUSH {R1, R2, R3}, in what order are the registers stored in memory?

A) R1, R2, R3 (from lowest to highest address)
B) R1, R2, R3 (lowest-numbered reg at lowest address; R1 pushed last)
C) R3, R2, R1 (from lowest to highest address)
D) No specific order; implementation-dependent

Question 12: Stack Pointer Behavior on PUSH

On a PUSH operation in Cortex-M (full descending stack), when is SP decremented?

A) After storing each register
B) Only once at the end
C) Before storing the first register
D) SP is incremented, not decremented

Question 13: Callee-Saved vs Caller-Saved

Which statement correctly describes caller-saved and callee-saved registers?

A) Caller-saved registers (R0-R3, R12) are preserved by the caller; callee-saved (R4-R11) by the callee
B) All registers are callee-saved
C) Caller-saved registers must be preserved by the callee
D) Only R0-R3 are ever used

Question 14: Register R4-R11 Preservation

If a subroutine modifies register R4-R11, what must the subroutine do?

A) The caller is responsible for saving them
B) The subroutine must save them (PUSH) and restore them (POP)
C) They don't need to be preserved
D) Save only odd-numbered registers

Question 15: Nested Subroutine Calls

Why must a subroutine save the Link Register (LR) when calling another subroutine?

A) The ARM architecture requires it
B) The inner BL instruction overwrites LR with a new return address, losing the original return path
C) To improve processor performance
D) It's optional and rarely needed

Question 16: Recursive Function Definition

What defines a recursive function?

A) A function that calls itself on smaller sub-problems to solve a larger problem, with a base case
B) A function that calls multiple other functions
C) A function that loops indefinitely
D) A function with no parameters

Question 17: Factorial Base Case

In a recursive factorial function, what is the base case?

A) factorial(n) = n × factorial(n-1)
B) factorial(0) = n
C) factorial(n) = 1 if n ≤ 1
D) factorial(n) = n!

Question 18: Stack Growth Direction

In a descending stack (used by Cortex-M), toward which direction does the stack grow?

A) Toward high memory addresses
B) Toward low memory addresses
C) Horizontally across the address bus
D) In a circular pattern

Question 19: PUSH {R1, R2} Memory Layout

After executing PUSH {R1, R2}, what is the memory layout (assuming R1=0x11111111, R2=0x22222222)?

A) [SP]: 0x11111111, [SP+4]: 0x22222222
B) [SP]: 0x22222222, [SP+4]: 0x11111111
C) [SP]: 0x22222222, [SP-4]: 0x11111111
D) Both registers stored at the same address

Question 20: Stack Pointer Preservation

According to the ARM EABI, what must be true about the Stack Pointer (SP/R13) after a subroutine returns?

A) SP can be any value
B) SP must have the same value as before the subroutine was called
C) SP is incremented by 4
D) SP is reset to 0

PART 2: COMPREHENSIVE ANSWER KEY

Answer 1: Link Register (LR) Primary Function

Correct Answer: B

The Link Register (R14) stores the return address when a subroutine is called. When BL executes, it saves PC + 4 in LR, allowing the subroutine to return via BX LR.

Answer 2: BL Instruction Behavior

Correct Answer: C

BL performs: (1) LR = PC + 4 (save return address), (2) PC = target_address (branch to subroutine).

Answer 3: Return from Subroutine

Correct Answer: B

BX LR sets PC = LR, returning control to the caller. Equivalent to POP {PC}.

Answer 4: ARM Procedure Call Standard - Argument Passing (≤4 args)

Correct Answer: A

ARM EABI specifies: Argument 1→R0, Argument 2→R1, Argument 3→R2, Argument 4→R3.

Answer 5: 64-bit Argument Passing

Correct Answer: C

64-bit values use two consecutive registers: R0:R1 (first argument) or R2:R3 (second argument).

Answer 6: Extra Arguments (>4)

Correct Answer: B

When more than 4 arguments exist, extra arguments are passed on the stack by the caller, who is also responsible for cleaning up.

Answer 7: Return Value (32-bit)

Correct Answer: A

32-bit return values are placed in R0. 64-bit values use R0:R1, and 128-bit values use R0-R3.

Answer 8: Cortex-M Stack Convention

Correct Answer: C

ARM Cortex-M uses full descending stack: SP points to the last item pushed (full), and stack grows toward low memory addresses (descending).

Answer 9: PUSH Instruction Equivalence

Correct Answer: B

PUSH is equivalent to STMDB SP! (Store Multiple, Decrement Before). DB means decrement SP first, then store.

Answer 10: POP Instruction Equivalence

Correct Answer: A

POP is equivalent to LDMIA SP! (Load Multiple, Increment After). Load first, then increment SP.

Answer 11: Register Order in PUSH

Correct Answer: B

Registers are sorted automatically by hardware. R1 (lowest number) is stored at the lowest memory address but pushed last. Final memory: [R1] [R2] [R3] (lowest to highest).

Answer 12: Stack Pointer Behavior on PUSH

Correct Answer: C

In full descending stack, SP is decremented BEFORE storing (pre-decrement). For PUSH {r4, r5}, SP is decremented by 8, then stores occur.

Answer 13: Callee-Saved vs Caller-Saved

Correct Answer: A

Caller-saved (R0-R3, R12): Caller preserves if needed. Callee-saved (R4-R11): Callee must preserve if modified.

Answer 14: Register R4-R11 Preservation

Correct Answer: B

If a subroutine modifies R4-R11 (callee-saved), it must save them at entry (PUSH) and restore at exit (POP).

Answer 15: Nested Subroutine Calls

Correct Answer: B

When foo() calls bar(), the BL bar instruction overwrites LR with bar's return address, destroying foo's return address. Solution: foo must save LR before the nested call.

Answer 16: Recursive Function Definition

Correct Answer: A

A recursive function calls itself on smaller sub-problems with a base case to terminate recursion.

Answer 17: Factorial Base Case

Correct Answer: C

Factorial base case: if (n ≤ 1) return 1. Prevents infinite recursion.

Answer 18: Stack Growth Direction

Correct Answer: B

Descending stack grows toward low memory addresses. SP decreases as items are pushed.

Answer 19: PUSH {R1, R2} Memory Layout

Correct Answer: C

[SP]: 0x22222222 (R2), [SP-4]: 0x11111111 (R1). R1 is at lower address, R2 at higher address.

Answer 20: Stack Pointer Preservation

Correct Answer: B

ARM EABI requires SP to have the same value after subroutine return as before the call. Maintains stack balance and prevents corruption.

Key Topics Summary

| Category | Concepts |
| --- | --- |
| Control Flow | BL, BX LR, return addresses, LR preservation |
| Parameter Passing | R0-R3 registers, stack arguments, 64-bit values |
| Stack Operations | Full descending, PUSH/POP, pre/post-decrement |
| Register Preservation | Caller-saved, callee-saved, EABI compliance |
| Advanced Topics | Nested calls, recursion, base cases |
| Memory Management | Stack layout, SP alignment, memory addressing |

Performance Analysis

Assessment Type: Knowledge-based multiple choice
Competency Areas:

- ARM Cortex-M instruction set and conventions

- Subroutine calling standards and protocols

- Stack memory management and operations

- Register preservation and function calling best practices

- Recursive function design and implementation